| Category: | General |
| --- | --- |
| Title: | Construction & Use of Recreational Fire Pits, Fire Bowls, Chimineas or Similar Appurtenances |

Reference: International Fire Code, 2024 Edition (2024 IFC); International Wildland Urban Interface Code, 2018 Edition (2018 IWUIC); National Fire Protection Association (NFPA).

307.1 General. A person shall not kindle or maintain or authorize to be kindled or maintained any open burning unless conducted and approved in accordance with Sections 307.1.1 through 307.5. (2024 IFC)

The following requirements are pertinent to the construction and use of recreational fire pits, fire bowls and similar appurtenances located within the boundaries of the Sedona Fire District.

- The Sedona Fire District shall not be liable for damages resulting from the installation, construction, use and/or misuse of fire pits.

- This interpretation does not absolve the user of said fire pits from complying with other applicable laws, rules, regulations or ordinances.

- Recreational burning shall comply with all seasonal fire restrictions (fire pits fueled solely by Liquefied Propane Gas (LPG) or Liquefied Natural Gas (LNG) shall be exempt from this provision).

- When a fire pit is in use, it shall be constantly attended by a competent person at least 18 years of age.  The attendant shall monitor the use of the fire pit until the fire is fully extinguished.  Disposal of ashes shall be in a metal container with a tight-fitting lid and the container shall be placed on a non-combustible surface.

- For wood burning fire pits, only dry seasoned firewood shall be used as fuel. Firewood shall not be stacked higher than 24 inches in the fire pit.  The burning of trash, yard debris, or construction materials shall be prohibited.

- The installation and use of fire pits require the permission of the landowner.

- The installation of permanent fire pits may require the issuance of a city or county permit.  Check with your local building and planning department for requirements.

- The installation and use of fire pits shall be in accordance with all manufacturer’s guidelines.

- The burn area of a fire pit shall not exceed 36 inches in diameter.

- Fire pits shall be no less than 25 feet from any building or structure. Exceptions shall be approved by Sedona Fire District Community Risk Reduction Division.

- A clear area, free of combustibles, no less than 36 inches wide shall be maintained in the immediate vicinity of the fire pit at all times.

- An approved means of fire suppression shall be available at all times that the fire pit is in use.  Approved means may include but are not limited to portable fire extinguisher, garden hose or other water supply, shovel and dirt.